- Answer to subquestions:

  - addi x5, x0, 2
add x6, x0, 3
addi x10, x5, 0
addi x11, x6, 0
jal x1, hishab
sw x10, 0(x18)


hishab:
addi x10, x10, 8
lw x28, 0(18)
slli x28, x28, 2
add x29, x18, x28
lw x28, 0(x29)
sub x11, x11, x28

loop:
bge x11, x10, exit
slli x30, x10, 2
addi x30, x30, x18
lw x11, 0(x30)
addi x10, x10, 1
beq x0, x0, loop

exit:
addi x10, x11, x0

jalr x0, 0(x1)

  - Instructions related to line 2 and line 3:
lw x28, 0(18)					i
slli x28, x28, 2					ii
add x29, x18, x28				iii	
lw x28, 0(x29)					iv
sub x11, x11, x28				v

loop:
bge x11, x10, exit				vi


Respective machine codes:

i.

| Machine Code | 000000000000 | 10010 | 010 | 11100 | 0000011 |
| --- | --- | --- | --- | --- | --- |
| Fields | imm | rs1 | funct3 | rd | opcode |

ii.

| Machine Code | 000010 | 000010 | 11100 | 001 | 11100 | 0000000 |
| --- | --- | --- | --- | --- | --- | --- |
| Fields | funct6 | imm | rs1 | funct3 | rd | opcode |

iii.

| Machine Code | 0000000 | 11100 | 10010 | 000 | 11101 | 0110011 |
| --- | --- | --- | --- | --- | --- | --- |
| Fields | funct7 | rs2 | rs1 | funct3 | rd | opcode |

iv.

| Machine Code | 000000000000 | 11101 | 010 | 11100 | 0000011 |
| --- | --- | --- | --- | --- | --- |
| Fields | imm | rs1 | funct3 | rd | opcode |

v.

| Machine Code | 0100000 | 11100 | 10011 | 000 | 11100 | 0110011 |
| --- | --- | --- | --- | --- | --- | --- |
| Fields | funct7 | rs2 | rs1 | funct3 | rd | opcode |

vi.

| Machine Code | 0 | 000000 | 01010 | 01011 | 101 | 1100 | 0 | 1100111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fields | imm[12] | imm [10:5] | rs2 | rs1 | funct3 | imm [4:1] | imm [11] | opcode |

  - In RISC-V, the funct3 field in a Load instruction specifies the type of load operation to perform. It tells the processor how many bytes to load from memory (such as a byte, halfword, word, or doubleword) and whether the value should be interpreted as signed or unsigned. This allows a single load opcode to support different data sizes and sign-extension behaviors, making the instruction more flexible and efficient.

  - The funct3 field is what tells the machine that this is a LD instruction and therefore a 64-bit load. Different values of funct3 indicate whether the instruction is loading a byte, halfword, word, or doubleword, and whether it should be sign-extended or zero-extended.