## Question - 1:

Consider the following code:

int hishab(int i, int j) {

i = i + 8;

j = j - a[a [0]];									-line 2

while (j < i) {									-line 3

j = a [i];

i += 1;

}

return j;

}

k = 2;

l = 3;

a [0] = hishab(k, l);

- Translate the following codes written in high level language into an instruction sequence written in RISC-V Assembly. Assume that the variables k and l are in registers X5 and X6 respectively. a is a word array, which has its base address stored in X18. 	[10]

- Convert the instructions related to line 2 and line 3 in the given code into machine code of RISC-V.    [6]

- What is the significance of the funct3 field in case of a Load instruction?   [2]

- Suppose you have converted LD X9, 10(X21) to its equivalent RISC-V machine code. Now based on which field machine will understand the size of the data being loaded into register X9?   [2]