- Floating point operations:

  - f3 = 4.0375
f4 = -0.0045
f5 = f4 - f3 
    = -0.0045 - 4.0375
    =  -0.00000001001001101111 - 100.0000100110011001101
    = -1.001001101111*2-8 - 1.000000100110011001101*22
    = -0.0000000001001001101111*22 - 1.000000100110011001101*22
    =  -1.0000001010110000001001*22
Fsub.s, here s means single precision. In single precision, bias = 127.
Biased exponent = 2+ 127 = 129
Biased exponent range = 1 to 28 - 2 = 1 to 254
129 falls in this range. So no overflow or underflow.

  - f3 = 4.0375
f4 = -0.0045
f5 = f3 / f4
    = 4.0375 / -0.0045
    = 100.0000100110011001101 / -0.00000001001001101111
    = 1.000000100110011001101*22 / -1.001001101111*2-8
    = 1.000000100110011001101*22 * (1/-1.001001101111*2-8)
    = 1.000000100110011001101*22 * -0.11011110001101*28
    = 1.000000100110011001101*22 * -1.1011110001101*27
    =  1.1100000010010011*29
Fdiv.s, here s means single precision. In single precision, bias = 127.
Biased exponent = 9+ 127 = 136
Biased exponent range = 1 to 28 - 2 = 1 to 254
136 falls in this range. So no overflow or underflow.

- The exponent field in IEEE 754 is stored as an unsigned integer (all bits are nonnegative). But real exponents can be both positive and negative. To handle both positive and negative exponents without needing a separate sign bit, IEEE 754 uses a bias. 
Suppose, actual exponent =  +5, in single precision format, bias = 127. So, biased exponent = 5 + 127 = 132. So positive exponents just shift up by the bias.
For a negative case, suppose, actual exponent = -5. So, biased exponent = -5 + 127 = 122. So negative exponents are still stored as a positive unsigned integer, thanks to the bias.

- Optimized multiplication enhances efficiency by minimizing the number of sequential steps required compared to traditional long multiplication. Rather than generating and adding each partial product one at a time, optimized multipliers employ parallel techniques, such as add-and-shift, to perform multiple operations simultaneously. This reduces the total execution time, as many calculations occur at once instead of sequentially. From a computational complexity perspective, traditional long multiplication grows quadratically with the size of the operands, whereas optimized methods lower both the number of steps and the required hardware cycles, resulting in significantly higher performance.

- In RISC-V, multiplying two 64-bit numbers produces a 128-bit result, but the mul instruction only keeps the lower 64 bits. The mulh instruction gives the upper 64 bits, which can be checked for overflow. If mulh returns zero (or the correct sign extension), the product fits within 64 bits; otherwise, overflow has occurred. For example, multiplying 2*109*3  10^9  gives a result that fits in 64 bits, so mulh = 0, but multiplying (263 - 1)*2 produces an overflow since mulh ≠ 0.

- flw     f1, 0(x13)        # load fp_array[0] -> f1 (a)

flw     f2, 4(x13)        # load fp_array[1] -> f2 (b)

addi x9, x0, 0           # c = 0 (x9 will hold c)

flt.s   x10, f1, f2       # x10 = 1 if (a < b), else 0

beq    x10, x0, ELSE         # if (a >= b) jump to ELSE

fcvt.w.s x19, f1          # c = (int) a

beq x0, x0, END

ELSE:

fcvt.w.s x10, f2          # c = (int) b

END: