- Suppose registers f3 and f4 have the respective values 4.0375 and -0.0045 stored in them. Perform the following in floating point arithmetic operations and find if there is any overflow/underflow.

  - fsub.s f5, f4, f3 [3]

  - fdiv.s f5, f3, f4   [3]

- Why is a bias added to the actual exponent in the IEEE 754 floating-point representation, and how does this affect the encoding of both positive and negative exponents? [2]

- How does optimized multiplication improve efficiency and performance compared to traditional long multiplication, especially in terms of speed and computational complexity? [2]

- “mulh” instruction can be used to check for 64-bit overflow - Explain how with necessary examples. [2]

- Convert the following code to RISC-V assembly language.
float a = fp_array [0];
float b = fp_array [1];

int c = 0;
if (a < b) {

c = (int) a;

} else {

c = (int) b;

}

Where fp_array is a floating point word array base address of which is stored in x13. [8]