Question 01

Design a 16 X 132 register file.

Note: You must label all the input and output pins [3]

Question 02
Why does the ALU control unit take the following bits as input?

Instruction[30, 14-12]  [2]

Question 03

Suppose you have a single instruction;

Add X21, X22, X23

Register read = 20s

Instruction Fetch = 20s

Memory Operation = 40s

Register write = 20s

ALU Operation = 40s

Suppose you have a pipelined setup and a single cycle setup, 
Which one will run the instruction faster? [3]

Question 04

Modify the BEQ datapath so that it works for BNE. [3]

Question 05

Write the control signals for the following instruction:

Ld X5, 10(X21);

i. ALU Src;

ii. RegWrite;

iii. MemRead;

iv. MemWrite;

v. MemtoReg. [5]

Question 06

Explain control hazards in your own words. [2]

Question 07

Why do we keep instruction and data memory separate? [2]