## R-Type Instructions

| Name | Instruction Syntax | Description | Example |
| --- | --- | --- | --- |
| add | add rd, rs1, rs2 | rd = rs1 + rs2 | add x18, x20, x21. [x20 register’s data added with x21’s data and stored in x18 register.] |
| subtract | sub rd, rs1, rs2 | rd = rs1 - rs2 | sub x18, x20, x21. [x21’s data is subtracted from x20 and stored in x18 register.] |
| Bitwise and | and rd, rs1, rs2 | rd = rs1 & rs2 | and x5, x6, x7. [bitwise and operation between x6 and x7 register and saved in x5.] |
| Bitwise or | or rd, rs1, rs2 | rd = rs1 ¦ rs2 | or x5, x6, x7. [bitwise or operation between x6 and x7 register and saved in x5.] |
| Bitwise xor | xor rd, rs1, rs2 | rd = rs1 ^ rs2 | xor x5, x6, x7. [bitwise xor operation between x6 and x7 register and saved in x5.] |
| Shift left logical | sll rd, rs1, rs2 | rd = rs1 << rs2 | sll x20, x21, x22. [x21’s data is shifted left by x22 and saved in x20.] The number of shifting times depends on x22. If x22 = 5 , then x21 will be shifted left by 5 times. |
| Shift right logical | srl rd, rs1, rs2 | rd = rs1 >> rs2 | srl x20, x21, x22. [x21’s data is shifted right by x22 and saved in x20.] The number of shifting times depends on x22. If x22 = 3 , then x21 will be shifted right by 3 times. |
| Multiply | mul rd, rs1, rs2 | rd = rs1 * rs2 | mul x20, x21, x22. [x21’s data is multiplied with x22’s data and only the lower 64 bits of the product are stored in x20] |
| Multiply high | mulh rd, rs1, rs2 | rd = rs1 * rs2 | mulh x20, x21, x22. [x21’s data is multiplied with x22’s data and only the upper 64 bits of the product are stored in x20.] Both rs1 and rs2 are signed |
| Multiply high unsigned | mulhu rd, rs1, rs2 | rd = rs1 * rs2 | mulhu x20, x21, x22. [x21’s data is multiplied with x22’s data, and only the upper 64 bits of the product is stored in x20] Both rs1 and rs2 are unsigned |
| Multiply high signed/unsigned | mulhsu rd, rs1, rs2 | rd = rs1 * rs2 | mulhsu x20, x21, x22. [x21’s data is multiplied with x22’s data, and only the upper 64 bits of the product is stored in x20.] rs1 is signed, and rs2 is unsigned. |

*NOTE: 	rs1 = source register 1, rs2 = source register 2, rd = destination register

All multiply instructions can only be used after the midterm.

## I-Type Instructions

| Name | Instruction Syntax | Description | Example |
| --- | --- | --- | --- |
| add immediate | addi rd, rs, imm | rd = rs1 + imm | addi x18, x20, 5. [x20 register’s data added with constant 5 and stored in x18 register.] If subtraction is needed, then 2s compliment of immediate is used for the negative number. |
| Bitwise and immediate | andi rd, rs, imm | rd = rs1 & imm | andi x5, x6, 45. [bitwise and operation between x6 and constant 45 and saved in x5] |
| Bitwise or immediate | ori rd, rs, imm | rd = rs1 ¦ imm | ori x5, x6, 22. [bitwise or operation between x6 and constant 22 and saved in x5.] |
| Bitwise xor immediate | xori rd, rs, imm | rd = rs1 ^ imm | xor x5, x6, 12. [bitwise xor operation between x6 and constant 12 and saved in x5.] |
| Shift left logical immediate | slli rd, rs, imm | rd = rs1 << imm | slli x20, x21, 4. [x21’s data is shifted left by 4 times and saved in x20.] |
| Shift right logical immediate | srli rd, rs, imm | rd = rs1 >> imm | srli x20, x21, 3. [x21’s data is shifted right by 3 times and saved in x2.] |
| Memory Instructions (LOAD) [Retrieve data from RAM to Register] |  |  |  |
| Load byte | lb rd, imm(rs1) | rd = M[rs1 + imm] [7:0] → 8 bits (immediate is sign extended) | lb x20, 1(x25). [Signed byte is loaded from memory to x20 register]. Each index of array is stored using 1 memory slot/location |
| Load byte unsigned | lbu rd, imm(rs1) | rd = M[rs1 + imm] [7:0] → 8 bits (immediate is extended with zero) | lbu x20, 1(x25).[Unsigned byte is loaded from memory to x20 register]. Each index of array is stored using 1 memory slot/location |
| Load half word | lh rd, imm(rs1) | rd = M[rs1 + imm] [15:0] → 16 bits (immediate is sign extended) | lh x20, 2(x25).[Signed half word is loaded from memory to x20 register]. Each index of array is stored using 2 memory slot/location |
| Load half word unsigned | lhu rd, imm(rs1) | rd = M[rs1 + imm] [15:0] → 16 bits (immediate is extended with zero) | lhu x20, 2(x25).[Unsigned half word is loaded from memory to x20 register]. Each index of array is stored using 2 memory slot/location |
| Load word | lw rd, imm(rs1) | rd = M[rs1 + imm] [31:0] → 32 bits (immediate is sign extended) | lw x20, 4(x25).[Signed word is loaded from memory to x20 register]. Each index of array is stored using 4 memory slot/location |
| Load word unsigned | lwu rd, imm(rs1) | rd = M[rs1 + imm] [31:0] → 32 bits (immediate is extended with zero) | lwu x20, 4(x25).[Unsigned word is loaded from memory to x20 register]. Each index of array is stored using 4 memory slot/location |
| Load double word | ld rd, imm(rs1) | rd = M[rs1 + imm] [63:0] → 64 bits (immediate is sign extended) | ld x20, 16(x25).[Signed double word is loaded from memory to x20 register]. Each index of array is stored using 8 memory slot/location |
| Load double word unsigned | ldu rd, imm(rs1) | rd = M[rs1 + imm] [63:0] → 64 bits (immediate is extended with zero) | ldu x20, 16(x25). [Unsigned double word is loaded from memory to x20 register]. Each index of array is stored using 8 memory slot/location |
| JALR |  |  |  |
| Jump and Link Register | jalr rd, imm(rs1) | PC = rs1 + imm | jalr x0, 0(x1). Returns control to the caller |

*NOTE:  	M[rs1 + imm] means data is loaded from memory(RAM). The target address is calculated as follows: Target address = rs1 + imm. Here, rs1 represents the base register and immediate represents the offset.

Each memory slot/location stores 8 bits. [RISC V is byte addressed]

- So 1 double word requires = 64/8 = 8 slots/locations of memory

- So 1 word requires = 32/8 = 4 slots/locations of memory

- So 1 half word requires = 16/8 = 2 slots/locations of memory

- So 1 byte requires = 8/8 = 1 slot/location of memory

## S - Type Instructions

| Memory Instructions (STORE) [Store data from Register to RAM] |  |  |  |
| --- | --- | --- | --- |
| Name | Instruction Syntax | Description | Example |
| Store byte | sb rs2, imm(rs1) | M[rs1 + imm] = rs2[7:0] → 8 bits (immediate is sign extended) | sb x20, 1(x25). [Signed byte is stored from x20 register to memory]. Each index of array is stored using 1 memory slot/location |
| Store half word | sh rs2, imm(rs1) | M[rs1 + imm] = rs2[15:0] → 16 bits (immediate is sign extended) | sh x20, 2(x25).[Signed half word is stored from x20 register to memory]. Each index of array is stored using 2 memory slot/location |
| Store word | sw rs2, imm(rs1) | M[rs1 + imm] = rs2[31:0] → 32 bits (immediate is sign extended) | sw x20, 4(x25).[Signed word is stored from x20 register to memory]. Each index of array is stored using 4 memory slot/location |
| Store double word | sd rs2, imm(rs1) | M[rs1 + imm] = rs2[63:0] → 64 bits (immediate is sign extended) | sd x20, 16(x25).[Signed double word is stored from x20 register to memory]. Each index of array is stored using 8 memory slot/location |

* RISC V does not have unsigned store instructions. Only Load has unsigned.

## SB - Type Instruction [Conditional Branching]

| Name | Instruction Syntax | Description | Example |
| --- | --- | --- | --- |
| Branch if equal | beq rs1, rs2, label | If rs1 == rs2: PC = PC + offset *jump to label Else: PC = PC + 4 | beq x5, x7, exit. [If x5 value and x7 value are equal, then jump to the exit label.] |
| Branch if not equal | bne rs1, rs2, label | If rs1 != rs2: PC = PC + offset *jump to label Else: PC = PC + 4 | bne x5, x7, exit. [If x5 value and x7 value are not equal, then jump to the exit label.] |
| Branch if less than (signed) | blt rs1, rs2, label | If rs1 < rs2: PC = PC + offset *jump to label Else: PC = PC + 4 | blt x20, x22, IF. [If rx20 value is less than x22 value, then jump to the IF label] x20 and x22 are signed values. |
| Branch if less than (unsigned) | bltu rs1, rs2, label |  | bltu x20, x22, IF. [If x20 value is less than x22 value,then jump to the IF label] x20 and x22 are unsigned values |
| Branch if greater or equal (signed) | bge rs1, rs2, label | If rs1 >= rs2: PC = PC + offset *jump to label Else: PC = PC + 4 | bge x25, x26, ELSE. [If x25 value is greater or equal to x26, then jump to ELSE label] x25 and x26 are signed values |
| Branch if greater or equal (unsigned) | bgeu rs1, rs2, label |  | bgeu x25, x26, ELSE. [If x25 value is greater or equal to x26, then jump to ELSE label] x25 and x26 are unsigned values |

*NOTE: label can be any placeholder name that defines a specific memory address.

Naming convention: Do not start with any numbers or special characters.

## UJ - Type Instruction [Unconditional Jump]

| Name | Instruction Syntax | Description | Example |
| --- | --- | --- | --- |
| Jump and Link | jal rd, procedureLabel | rd = PC + 4 PC = PC + offset | jal x1, functionA. Jumps to the function block. Saves the return address in x1. |

## U - Type Instruction [Upper immediate]

| Name | Instruction Syntax | Description | Example |
| --- | --- | --- | --- |
| Load upper immediate | lui rd, immediate | rd[31:12] = imm | lui x20, 976. copies the 20-bit immediate to the upper 20 bits of x20[31:12] |

References:
Cornell 	RISC V