# Types of Hazard

For our Syllabus, we need to know the three hazard types, but only explore the resolution of Structural and Data Hazards.

- Structural Hazard: Occurs when the system is trying to use the same resource multiple times in the same clock cycle.

Example:

| C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IF | ID | EX | MEM | WB |  |  |  |
|  | IF | ID | EX | MEM | WB |  |  |
|  | IF | ID | EX | MEM | WB |  |  |

- The red rows would cause structural hazard due to same resource being used twice in the same clock cycle.

- Data memory and instruction memory are split so that the IF and MEM stages can exist together in the same cycle without causing a hazard.

IF stage for instruction memory and MEM stage for Data memory.

- Data Hazard: Occurs when an instruction depends on the completion of data access by a previous instruction.

Example: add x19, x0, x1

Sub x2, x19, x3

|  | C1 | C2 | C3 | C4 | C5 | C6 |
| --- | --- | --- | --- | --- | --- | --- |
| add | IF | ID | EX | MEM | WB |  |
| sub |  | IF | ID | EX | MEM | WB |

Clock cycle 3 would cause a data hazard. Because in the sub instruction, the x19 register’s data is decoded, but the data from the previous add instruction is not yet updated in the register file. It is updated in the WB stage in clock cycle 5.

- Control Hazard: Occurs when the pipeline does not know what will be the next instruction address. It depends on the previous instruction’s completion.

Example:  Branch type instructions. In these instruction types, the next instruction address can have two options.
	PC + Offset if the branch condition is true.

PC + 4 if the branch condition is false.

Until the instruction is fully executed(EX/ ALU), the pipeline will not know what control signals to give. This is known as Control Hazard.

# Stalling

Question: Identify and resolve the data hazards for the following sequence of instructions using only Stalling.

- Add X17, X18, X20

- Sub X20, X17, X5

- Sd X25, 20(X19)

- Slli X21, X20, 4

- Ld X18, 16(X21)

Solution:

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 | C13 | C14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
|  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |
|  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |
| 2 |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| 3 |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |
|  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |
| 4 |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |
|  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |
| 5 |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

# stalling+forwarding

# Stalling + Forwarding

Example-1:

- ld	x1, 0(x0)

- ld	x2, 8(x0)

- add	x3, x1, x2

- sd	x3, 24(x0)

- ld	x4, 16(x0)

- add	x5, x1, x4

- sd	x5, 32(x0)

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 | C13 | C14 | C15 | C16 | C17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |
| 7. |  |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

Example-2:

- Add x3, x1, x2 	# NO DATA HAZARD

- Ld x20, 16(x3)	# X3 is read and it was updated before (Solve using Forwarding from EX/MEM to EX)

- Sub x5, x20, x0	# X5 is read and it was written to before (Solve using one Stall + Forward from MEM/WB to EX)

- Sw x3, 8(x5)	# X5 is read again and it was updated again before (Solve using Forwarding from EX/MEM to EX) [X3 is also read in ID stage(C6), but its last update was completed by C5]

- Addi x22, x5, -5	# X5 is read again and we have to make sure it is getting the most updated value (Solve using Forwarding from MEM/WB to EX)

- Lb x17, 2(x23)	# NO DATA HAZARD

- Sd x17, 0(x28)	# X17 is read which was written to before. (Solve using two Stalls to align WB stage of Lb instruction to ID stage of Sd instruction)

- And x6, x26, x31   #NO DATA HAZARD

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 | C13 | C14 | C15 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |

# Stall+Forward+Code Scheduling

# Stalling + Forwarding + Code Scheduling

Example-1:

- ld	x1, 0(x0)

- ld	x2, 8(x0)

- ld	x4, 16(x0)

- add	x3, x1, x2

- sd	x3, 24(x0)

- add	x5, x1, x4

- sd	x5, 32(x0)

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 | C13 | C14 | C15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |
| 2. |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
| 3. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
| 4. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
|  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |
|  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |
| 5. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| 6. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |
|  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |
| 7. |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

Example-2:

- Add x3, x1, x2 	# NO DATA HAZARD

- Ld x20, 16(x3)	# X3 is read and it was updated before (Solve using Forwarding from EX/MEM to EX)

6.  Lb x17, 2(x23)	# NO DATA HAZARD, so rearrange it to replace the first stall.

- Sub x5, x20, x0	# X5 is read and it was written to before (Solve using one Stall + Forward from MEM/WB to EX)

- Sw x3, 8(x5)	# X5 is read again and it was updated again before (Solve using Forwarding from EX/MEM to EX) [X3 is also read in ID stage(C6), but its last update was completed by C5]

- Addi x22, x5, -5	# X5 is read again and we have to make sure it is getting the most updated value (Solve using Forwarding from MEM/WB to EX)

8. And x6, x26, x31    # NO DATA HAZARD, so rearrange it to replace the first stall.

7. Sd x17, 0(x28)	# X17 is read which was written to before. (Solve using two Stalls to align WB stage of Lb instruction to ID stage of Sd instruction)

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 | C13 | C14 | C15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |
| 6. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |
|  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |