Q. Simulate and show how the cache table changes when the following memory addresses are accessed in the given order.

Given Cache size = 4 blocks. Block size = 2 bytes.

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | N Y | 10 | mem[10000] |
|  | 1 | N Y | 11 01 | Mem[11001] mem[01001] |
| 01 | 0 | N |  |  |
|  | 1 | N |  |  |
| 10 | 0 | N |  |  |
|  | 1 | N Y | 10 | mem[10101] |
| 11 | 0 | N |  |  |
|  | 1 | N |  |  |

Memory address access history:

| Memory Address | Cache hit/miss |
| --- | --- |
| 25 | miss |
| 16 | miss |
| 21 | miss |
| 25 | hit |
| 9 | miss |
| 21 | hit |

Solution:

Index size = log2(4) = 2 bit

Offset size = log2(2) = 1 bit

Highest address given in the question is 25 which requires 5-bit in binary.

Hence total memory address length will be considered as 5-bit

Tag size = 5 - 2 - 1 = 2 bit

- 25 = 11001 	Index = 00, Offset = 1, Tag = 11

- 16 = 10000	Index = 00, Offset = 0, Tag = 10

- 21 = 10101	Index = 10, Offset = 1, Tag = 10

- 25 = 11001	Index = 00, Offset = 1, Tag = 11

- 9 =   01001	Index = 00, Offset = 1, Tag = 01

- 21 = 10101	Index = 10, Offset = 1, Tag = 10

Note: Show all your workings.