BRAC University

CSE340: Computer Architecture

Quiz#1

Duration : 20 Minutes

Total Marks : 15	Set: Abacus

ID:                                     Name: 						        Sec:

- Here is a set of instructions for an Intel i7 processor (e.g., Skylake or newer) using Intel syntax for checking if a number is prime.

| _start: mov rax, [num] cmp rax, 2 jl not_prime mov rcx, 2 check_loop: mov rdx, 0 mov rbx, rcx div rbx cmp rdx, 0 je not_prime inc rcx mov rbx, rcx imul rbx, rbx cmp rbx, rax jle check_loop jmp end not_prime: mov byte [is_prime], 0 end: mov rax, 60 xor rdi, rdi syscall | 10s Instruction Type; CPI. mov; 1. cmp; 1. jl, je, jle, jmp; 1. div; 30. imul; 1. inc, xor; 2. syscall; 1. |
| --- | --- |

  - Find the number of instructions in the above code. [2]

  - Find the average CPI. [2]

  - Find the clock cycle time. [2]

  - Find the time to execute this program. [4]

- A CPU has a MIPS rating of 6. A program consists of: 10 million instructions, of which 60% are arithmetic (CPI = 2), 30% are memory access (CPI = 3), and 10% are control (CPI = 1).

Determine:

a) Average CPI [1]

b) Clock frequency of the processor [2]

c) Execution time of the program [2]

BRAC University

CSE340: Computer Architecture

Quiz#1

Duration : 20 Minutes

Total Marks : 15	Set: Byte

ID:                                     Name: 						        Sec:

- Here is a set of instructions for an Intel i7 processor (e.g., Skylake or newer) using Intel syntax for checking if a number is prime.

| _start: mov rsi, 0 mov rax, 0 mov rbx, 1 mov rcx, [n] mov rdi, fibs generate_loop: cmp rsi, rcx jge done mov [rdi + rsi*8], rax mov rdx, rax add rax, rbx mov rbx, rdx inc rsi jmp generate_loop done: mov rax, 60 xor rdi, rdi syscall | 10s Instruction Type; CPI. mov; 1. cmp; 1. jl, je, jge, jmp; 1. div; 30. imul; 1. inc, xor; 2. syscall; 1. |
| --- | --- |

  - Find the number of instructions in the above code. [2]

  - Find the average CPI. [2]

  - Find the clock cycle time. [2]

  - Find the time to execute this program. [4]

- A CPU has a MIPS rating of 5. A program consists of: 20 million instructions, of which 60% are arithmetic (CPI = 3), 30% are memory access (CPI = 5), and 10% are control (CPI = 2).

Determine:

a) Average CPI [1]

b) Clock frequency of the processor [2]

c) Execution time of the program [2]