BRAC University

CSE340: Computer Architecture

Makeup Quiz

Duration : 20 Minutes

Total Marks : 15

ID:                                     Name: 						        Sec:

1. Design a 128X1024 register file.[5]

2. The following table shows the different stages involved in executing instructions and the corresponding durations for each stage:

| stages | Instruction Fetch | Register Read | ALU Op | Memory Access | Register Write |
| --- | --- | --- | --- | --- | --- |
| Duration | 50ps | 10ps | 30ps | 20ps | 10ps |

Given the above durations, determine the total time required to complete each of the following instructions: [5]

|  | Instructions | Time to complete each instruction |
| --- | --- | --- |
| i. | ADD X21, X22, X23 |  |
| ii | ADDi X21, X22, 5 |  |
| iii. | LD X21, 22(X20) |  |
| iv. | SD X5, 22(X23) |  |
| v | BEQ X6, X8, End |  |

Suppose the above instructions are being run in a single-cycle datapath.

a.  Determine the clock period of this system.                                                    [1]

Answer:

b. Calculate the number of clock cycles needed to execute this instruction set. [2]

Answer:

c. What would be the total time to run this instruction sequence?                       [2]

Answer: