Set A:

- lui x22, BACADh
addi x22, x22, 1E0h

- RISC-V assembly:
add x5, x20, x19			Memory location of B[i]
lb x6, 0(x5)				Loading value of B[i]
add x28, x6, x18			Memory location of A[ B [ i ] ]
lb x29, 0(x28)				Loading value of B[ A [ i ] ]

add x28, x20, x18			Memory location of A[i]
lb x30, 0(x28)				Loading value of A[i]
add x31, x30, x19			Memory location of B[ A [ i ] ]
lb x7, 0 (x31)				Loading value of B[ A [ i ] ]

bne x29, x7,else1			Checking if B[ A [ i ] ] != A[ B [ i ] ]
bge x30, x6, else2			Checking if A [i] >= B [i]
addi x30, x30, 1			Implementing A [i] ++
sb x30, 0(x28)				Storing A [i] back to the array memory location 
beq x0, x0, exit			Exiting the remaining elses

else2:					Inner else
addi x6, x6, 1				Implementing B [i] ++
sb x6, 0(x5)				Storing B [i] back to the array memory location 
beq x0, x0, exit			Exiting the remaining elses

else1: 					Outer else
sb x6, 0(x28)				Implementing A [i] = B [i]; current value of B[i] in x6 
					and memory location of A [i] in x28.
exit:

- 2 Line Backward jump in assembly language.
So, 2 * 4 = 8 Line Backward jump in memory location.
13 bit binary of 8 is: 0000000001000
As it is a backward jump we need to convert to 2s complement.
1111111110111
	       +1
—-----------------

1111111111000

Now converting to machine code:

| Machine Code | 1 | 111111 | 00000 | 00101 | XXX | 1100 | 1 | XXXXXXX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fields | Imm [12] | Imm [10:5] | rs2 | rs1 | funct3 | Imm [4:1] | Imm [12] | Opcode |

Machine Code :  11111110000000101XXX11001XXXXXXX

Set B:

- lui x22, CADADh
addi x22, x22, 1E0h

- RISC-V assembly:
add x5, x20, x19			Memory location of B[i]
lb x6, 0(x5)				Loading value of B[i]
add x28, x6, x18			Memory location of A[ B [ i ] ]
lb x29, 0(x28)				Loading value of B[ A [ i ] ]

add x28, x20, x18			Memory location of A[i]
lb x30, 0(x28)				Loading value of A[i]
add x31, x30, x19			Memory location of B[ A [ i ] ]
lb x30, 0 (x31)				Loading value of B[ A [ i ] ]

beq x29, x30,else1			Checking if B[ A [ i ] ] == B[ A [ i ] ]
blt x30, x6, else2			Checking if A [i] < B [i]
addi x30, x30, -1			Implementing A [i] --
sb x30, 0(x28)				Storing A [i] back to the array memory location 
beq x0, x0, exit			Exiting the remaining elses

else2:					Inner else
addi x6, x6, -1				Implementing B [i] --
sb x6, 0(x5)				Storing B [i] back to the array memory location 
beq x0, x0, exit			Exiting the remaining elses

else1: 					Outer else
sb x30, 0(x5)				Implementing B [i] = A [i]; current value of A[i] in
					x30 and memory location of B [i] in x5.

- 2 Line Backward jump in assembly language.
So, 2 * 4 = 8 Line Backward jump in memory location.
13 bit binary of 8 is: 0000000001000
As it is a backward jump we need to convert to 2s complement.
1111111110111
	       +1
—-----------------

1111111111000

Now converting to machine code:

| Machine Code | 1 | 111111 | 00000 | 00110 | XXX | 1100 | 1 | XXXXXXX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fields | Imm [12] | Imm [10:5] | rs2 | rs1 | funct3 | Imm [4:1] | Imm [12] | Opcode |

Machine Code :  11111110000000101XXX11001XXXXXXX