BRAC University

CSE340: Computer Architecture

Quiz#2

Duration : 20 Minutes

Total Marks : 15	Set: Arithmetic

ID:                                     Name: 						        Sec:

- Load the number 0xBACAD1E0 in the register x22.   [2]

- Translate the following codes written in high level language into an instruction sequence written in RISC-V Assembly. Assume that the variable i is stored in register X20 and the base address of the byte arrays a and b are in X18 and X19 respectively.   [10]

if (A [B [i] ]== B [A [i] ]) {

if (A [i] < B [i]) {

A [i] ++;

} else {

B [i] ++;

}

} else {

A [i] = B [i];

}

- Find the machine code of only the selected(bold) line:

addi x5, x0, 5

addi x6, x0, 0

loop:

add  x6, x6, x5

addi x5, x5, -1

bne  x5, x0, loop

addi x10, x6, 0

addi x17, x0, 10

The starting address of the first instruction is 0x5000. Use X for the fields that are not given.    [3]

BRAC University

CSE340: Computer Architecture

Quiz#2

Duration : 20 Minutes

Total Marks : 15	Set: Boolean

ID:                                     Name: 						        Sec:

- Load the number 0xCADAD1E0 in the register x20.    [2]

- Translate the following codes written in high level language into an instruction sequence written in RISC-V Assembly. Assume that the variable i is stored in register X20 and the base address of the byte arrays a and b are in X18 and X19 respectively.    [10]

if (A [B [i] ] !=  B [A [i] ]) {

if (A [i] ≥ B [i]) {

A [i]--;

} else {

B [i] --;

}

} else {

B [i] = A  [i];

}

- Find the machine code of only the selected(bold) line:

addi x5, x0, 3

addi x6, x0, 4

addi x7, x0, 0

loop:

add  x7, x7, x5

addi x6, x6, -1

bne  x6, x0, loop

addi x10, x7, 0

addi x17, x0, 10

The starting address of the first instruction is 0x7000. Use X for the fields that are not given.    [3]