Set A:

- Control Signal Values:

| Control Signal | Instruction i bne x5, x6, L0 | Instruction ii ori x6, x6, 1 | Explanation |
| --- | --- | --- | --- |
| Branch | 1 | 0 | i. As it is a branch instruction, branch signal is high(1) ii. Not a branch instruction, branch signal is low(0) |
| Mem Read | 0 | 0 | i & ii. No memory read operation ongoing (both work only on registers) |
| MemtoReg | 0/1/X | 0 | i. This control signal selects which data will flow to write register, as branch instruction will not write to any register, this value is don’t care(X) ii. The data being written to the register(x6) is an output of the ALU, hence 0. |
| ALUOp | 01 | 10 | From the given table in the question |
| Mem Write | 0 | 0 | i & ii. No memory write operation ongoing (both work only on registers) |
| ALU Src | 0 | 1 | i. This control signal selects from where the second data to the ALU will flow, in branch instruction there are two source registers, so it will flow from a register, hence 0 ii. In ori instruction the second data is an immediate value(1), hence immediate value will be selected resulting in ALUSrc = 1. |
| Reg Write | 0 | 1 | i. No register write operation in a branch instruction ii. Destination register(x6) is present, so write operation is happening, hence 1. |

- Data hazard solution:
Lets number the instructions first:
i.  lw x5, 36(x6)
ii. lw x6, 40(x5)
iii. lw x7, 33(x6)
iv. sll x7, x6, 2
v. sub x5, x7, x6
vi. addi x5, x5, 2
vii. srl x5, x7, 2

Now, find the data hazards. 
- Between instruction i & ii, there is a hazard as i is writing to x5 and ii is reading from x5.
- Between instruction ii & iii, there is a hazard as ii is writing to x6 and iii is reading from x6.
- Between instruction iii & iv, there is no hazard as iv is reading from x6 and it has been written before.
- Between instruction iv & v, there is a hazard as iv is writing to x7 and v is reading from x7.
- Between instruction v & vi, there is a hazard as v is writing to x5 and vi is reading from x5.
- Between instruction vi & vii, there is no hazard as vii is reading from x7 and it has been written before.

  - Using only stalling (Total cycles = 19)

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |  |  |
| ii. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |
| iii. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
| iv. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
| v. |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |
| vi. |  |  |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

  - Using stalling and forwarding  (Total cycles = 13)

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |
|  |  |  |  |  | ↘ |  |  |  |  |  |  |  |  |
| ii. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
|  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
|  |  |  |  |  |  |  | ↘ |  |  |  |  |  |  |
| iii. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| iv. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  | ↘ |  |  |  |  |
| v. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |
|  |  |  |  |  |  |  |  |  |  | ↘ |  |  |  |
| vi. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
|  |  |  |  |  |  |  |  |  |  |  | ↘ |  |  |
|  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Using stalling, forwarding and code scheduling  (Total cycles = 13)
Here, in the two empty slots in the upper table (bubble rows), for code scheduling we need to put any instruction from below that is not dependent. But we see that all the later instructions are dependent on the previous ones. Thus we can not optimize more by code scheduling.

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |
|  |  |  |  |  | ↘ |  |  |  |  |  |  |  |  |
| ii. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
|  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
|  |  |  |  |  |  |  | ↘ |  |  |  |  |  |  |
| iii. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| iv. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  | ↘ |  |  |  |  |
| v. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |
|  |  |  |  |  |  |  |  |  |  | ↘ |  |  |  |
| vi. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
|  |  |  |  |  |  |  |  |  |  |  | ↘ |  |  |
|  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Set B:

- Control Signal Values:

| Control Signal | Instruction i bge x18, x19, P0 | Instruction ii xori x9, x9, 1 | Explanation |
| --- | --- | --- | --- |
| Branch | 1 | 0 | i. As it is a branch instruction, branch signal is high(1) ii. Not a branch instruction, branch signal is low(0) |
| Mem Read | 0 | 0 | i & ii. No memory read operation ongoing (both work only on registers) |
| MemtoReg | 0/1/X | 0 | i. This control signal selects which data will flow to write register, as branch instruction will not write to any register, this value is don’t care(X) ii. The data being written to the register(x9) is an output of the ALU, hence 0. |
| ALUOp | 01 | 10 | From the given table in the question |
| Mem Write | 0 | 0 | i & ii. No memory write operation ongoing (both work only on registers) |
| ALU Src | 0 | 1 | i. This control signal selects from where the second data to the ALU will flow, in branch instruction there are two source registers, so it will flow from a register, hence 0 ii. In ori instruction the second data is an immediate value(1), hence immediate value will be selected resulting in ALUSrc = 1. |
| Reg Write | 0 | 1 | i. No register write operation in a branch instruction ii. Destination register(x9) is present, so write operation is happening, hence 1. |

- Data hazard solution:
Lets number the instructions first:
i.  lw x18, 36(x19)
ii. lw x19, 40(x18)
iii. lw x20, 33(x19)
iv. sll x20, x19, 2
v. sub x18, x20, x19
vi. addi x18, x18, 2
vii. srl x18, x20, 2

Now, find the data hazards. 
- Between instruction i & ii, there is a hazard as i is writing to x18 and ii is reading from x18.
- Between instruction ii & iii, there is a hazard as ii is writing to x19 and iii is reading from x19.
- Between instruction iii & iv, there is no hazard as iv is reading from x19 and it has been written before.
- Between instruction iv & v, there is a hazard as iv is writing to x20 and v is reading from x20.
- Between instruction v & vi, there is a hazard as v is writing to x18 and vi is reading from x18.
- Between instruction vi & vii, there is no hazard as vii is reading from x20 and it has been written before.

  - Using only stalling (Total cycles = 19)

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |  |  |
| ii. |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |  |  |
| iii. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
| iv. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
| v. |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |
| vi. |  |  |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

  - Using stalling and forwarding  (Total cycles = 13)

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |
|  |  |  |  |  | ↘ |  |  |  |  |  |  |  |  |
| ii. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
|  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
|  |  |  |  |  |  |  | ↘ |  |  |  |  |  |  |
| iii. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| iv. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  | ↘ |  |  |  |  |
| v. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |
|  |  |  |  |  |  |  |  |  |  | ↘ |  |  |  |
| vi. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
|  |  |  |  |  |  |  |  |  |  |  | ↘ |  |  |
|  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Using stalling, forwarding and code scheduling  (Total cycles = 13)
Here, in the two empty slots in the upper table (bubble rows), for code scheduling we need to put any instruction from below that is not dependent. But we see that all the later instructions are dependent on the previous ones. Thus we can not optimize more by code scheduling.

| Ins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i. | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
|  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |  |  |
|  |  |  |  |  | ↘ |  |  |  |  |  |  |  |  |
| ii. |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
|  |  |  |  | 💭 | 💭 | 💭 | 💭 | 💭 |  |  |  |  |  |
|  |  |  |  |  |  |  | ↘ |  |  |  |  |  |  |
| iii. |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| iv. |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |
|  |  |  |  |  |  |  |  |  | ↘ |  |  |  |  |
| v. |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |
|  |  |  |  |  |  |  |  |  |  | ↘ |  |  |  |
| vi. |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
|  |  |  |  |  |  |  |  |  |  |  | ↘ |  |  |
|  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |
| vii. |  |  |  |  |  |  |  |  |  |  |  |  |  |