BRAC University

CSE340: Computer Architecture

Quiz#3

Duration : 20 Minutes

Total Marks : 15	Set: Arithmetic

ID:                                     Name: 						        Sec:

- Deduce the values of the following control signals for the following instructions:            			  [14 * 0.5 = 7]

| - Branch - MemRead - MemtoReg - ALUOp - MemWrite - ALUSrc - RegWrite i. bne x5, x6, L0 ii. ori x6, x6, 1 |  |
| --- | --- |

- Consider the following instructions:

lw x5, 36(x6)
lw x6, 40(x5)
lw x7, 33(x6)
sll x7, x6, 2
sub x5, x7, x6
addi x5, x5, 2
srl x5, x7, 2

  - If you only use Stall to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [2]

  - If you only use Stall and Forwarding to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [3]

  - If you only use Stall, Forwarding and load ordering/code scheduling to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [3]

N.B: For each of the above questions you need to show the datapaths for the instructions.

BRAC University

CSE340: Computer Architecture

Quiz#2

Duration : 20 Minutes

Total Marks : 15	Set: Boolean

ID:                                     Name: 						        Sec:

- Deduce the values of the following control signals for the following instructions:            [14 * 0.5 = 7]

| - Branch - MemRead - MemtoReg - ALUOp - MemWrite - ALUSrc - RegWrite i. bge x18, x19, P0 ii. xori x9, x9, 1 |  |
| --- | --- |

- Consider the following instructions:

lw x18, 36(x19)
lw x19, 40(x18)
lw x20, 33(x19)
sll x20, x19, 2
sub x18, x20, x19
addi x18, x18, 2
srl x18, x20, 2

  - If you only use Stall to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [2]

  - If you only use Stall and Forwarding to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [3]

  - If you only use Stall, Forwarding and load ordering/code scheduling to overcome the data hazard how many clock cycles would you need for executing the above code sequence? [3]

N.B: For each of the above questions you need to show the datapaths for the instructions.