- Index bit size = log2(cache size in blocks) = log2(4) = 2
Offset bit size = log2(block size in bytes) = log2(2) = 1 [16 bits = 2 bytes]
Now how do we find the total memory address size?
In the given table, we can see that the index bit size and offset bit size matches our calculation and there are some given tag bits which are of size 3. We can deduce from here that the total memory address size will be tag bit size + index bit size + offset bit size which is 3 + 2 + 1 = 6.
Another way to find this out is by figuring out how many bits do we need to represent the largest address given, which is 39. Binary representation of 39 is 100111, which is of length 6. Thus, total memory address size is 6 and tag bit size = 6 - 2- 1 = 3. [We considered the maximum address because memory address size can not vary in a single system and the highest address is the longest address.]

  - Memory address = 4 = 000100
Tag bits = 000
Index bits = 10
Offset bit = 0
In the given table, we first match the index column. As we can see, in the index = 10 row, both the valid bits are N(No) meaning it does not have any data. So, it is a cache miss.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | N |  |  |
|  | 1 | N |  |  |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | N |  |  |
|  | 1 | Y | 100 | Data |

  - Memory address = 39 = 100111
Tag bits = 100
Index bits = 11
Offset bit = 1
In the given table, we first match the index column. As we can see, in the index = 11 row, a valid bit is Y and another is N. So we need to match the offset bit. In the offset bit = 1 row, we can see the valid bit is Y. Then we match the tag bits, which is 100 and it also matches. Thus, it is a cache hit.
Table after the access:
[We are updating offset bit = 0 row because the whole cache block gets updated together.]

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | N |  |  |
|  | 1 | N |  |  |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

  - Memory address = 5 = 000101
Tag bits = 000
Index bits = 10
Offset bit = 1
In the given table, we first match the index column. As we can see, in the index = 10 row, both the valid bits are Y(Yes). Then we match the tag bits = 000 which also matches. So this is a cache hit.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | N |  |  |
|  | 1 | N |  |  |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

  - Memory address = 4 = 000100
Tag bits = 000
Index bits = 10
Offset bit = 0
In the given table, we first match the index column. As we can see, in the index = 10 row, both the valid bits are Y. Then we match the tag bits = 000 which also matches. So this is a cache hit.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | N |  |  |
|  | 1 | N |  |  |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

  - Memory address = 27 = 011011
Tag bits = 011
Index bits = 01
Offset bit = 1
In the given table, we first match the index column. As we can see, in the index = 01 row, both the valid bits are N(No) meaning it does not have any data. So, it is a cache miss.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | Y | 011 | Data |
|  | 1 | Y | 011 | Data |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

  - Memory address = 12 = 001100
Tag bits = 001
Index bits = 10
Offset bit = 0
In the given table, we first match the index column. As we can see, in the index = 10 row, both the valid bits are Y(Yes). Then we match the tag bits = 001 which does not match. So this is a cache miss.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | Y | 011 | Data |
|  | 1 | Y | 011 | Data |
| 10 | 0 | Y | 001 | Data |
|  | 1 | Y | 001 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

  - Memory address = 5 = 000101
Tag bits = 000
Index bits = 10
Offset bit = 1
In the given table, we first match the index column. As we can see, in the index = 10 row, both the valid bits are Y(Yes). Then we match the tag bits = 000 which does not match. So this is a cache miss.
Table after the access:

| Index | Offset | Valid | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | 0 | Y | 101 | Data |
|  | 1 | N |  |  |
| 01 | 0 | Y | 011 | Data |
|  | 1 | Y | 011 | Data |
| 10 | 0 | Y | 000 | Data |
|  | 1 | Y | 000 | Data |
| 11 | 0 | Y | 100 | Data |
|  | 1 | Y | 100 | Data |

Access History:

| Memory Address | Hit/Miss |
| --- | --- |
| 4 | Miss |
| 39 | Hit |
| 5 | Hit |
| 4 | Hit |
| 27 | Miss |
| 12 | Miss |
| 5 | Miss |

- Cache miss time = 85ns
Cache hit time = 15ns
Total access time for 1 is = (hit # * hit time) + (miss # * miss time) = (3 * 15 + 4 * 85) = 385ns

- Hit ratio = 3/7

MCQ

- B (Register > L1 Cache > L2 Cache > L3 Cache > RAM > Secondary Storage)

- D (See slide for reference)

- C (See slide for reference)

- B (See slide for reference)

- C (See slide for reference)