BRAC University

CSE340: Computer Architecture

Quiz#4

Duration : 20 Minutes

Total Marks : 15	Set: Hiererchy

ID:                                     Name: 						        Sec:

Given, cache size = 4 blocks and main memory block size = 16 bits.

- Simulate the given cache table for each of the memory addresses accessed by the CPU using a direct-mapped cache. For each address, you must show the index, tag and offset bits in your workings. Mention whether each access is a hit or miss. 		 [7]

Current State of the Cache Table:

- Assuming each cache hit takes 15 ns and each cache miss takes 85 ns, calculate the total access time. 								[2]

- What is the hit ratio for the given access history? 				[1]

Choose the correct option. 1 mark each.

1. Which memory in the hierarchy is the fastest?	[1]
 A. Disk      B. Registers      C. Main Memory     D. Cache
 Answer: __________

2.  Which type of locality refers to reusing the same data in a short time?      [1]
 A. Associative       B. Spatial        C. Sequential        D. Temporal                                                    Answer: __________

3. If the cache block size increases, keeping the cache size constant, what is likely to increase?[1]
 A. No. of cache lines     B. Cache conflicts         C. Cache hit ratio              D. Cache access time
 Answer: __________

4. Which policy reduces write traffic to main memory the most? 	[1]

A. Write-through with write-allocate   	     B. Write-back with write-allocate                            C. Write-through with no-write-allocate             D. Write-back with no-write-allocate
 Answer: __________

5. Which statement is TRUE for SRAM vs DRAM?       [1]
 A. SRAM is slower but cheaper than DRAM
 B. SRAM needs periodic refresh while DRAM does not
 C. SRAM mainly uses transistors, DRAM uses capacitors
 D. DRAM is mainly used in cache
 Answer: _________